HDLC Configuration in Cisco Packet Tracer

HDLC (High-Level Data Link Control) is a Data Link Layer protocol used for communication between routers over a point-to-point serial link. In Cisco Packet Tracer, you can configure HDLC between two routers and verify connectivity using the ping command.

1. Objective

To configure and verify HDLC encapsulation between two routers in Cisco Packet Tracer.

2. Network Topology

Create the following topology:

PC0 -------- R1 ================= R2 -------- PC1

S0/0/0           S0/0/0

Here, ==== represents a serial connection between the routers.

Devices required

- 2 Routers (e.g., Cisco 2911 with suitable serial modules, or another router model supporting serial interfaces)

- 2 PCs

- 2 Copper straight-through cables

- 1 Serial DCE/DTE cable

Important: If your router does not have serial interfaces, switch it off in the Physical tab, install a compatible serial module such as WIC-2T, and switch it back on.

3. IP Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC0 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 |
| R1 | GigabitEthernet0/0 | 192.168.1.1 | 255.255.255.0 |
| R1 | Serial0/0/0 | 10.0.0.1 | 255.255.255.252 |
| R2 | Serial0/0/0 | 10.0.0.2 | 255.255.255.252 |
| R2 | GigabitEthernet0/0 | 192.168.2.1 | 255.255.255.0 |
| PC1 | FastEthernet0 | 192.168.2.2 | 255.255.255.0 |

Default gateways:

- PC0: 192.168.1.1

- PC1: 192.168.2.1

4. Configure PC IP Addresses

For PC0:

- Click PC0.

- Open Desktop → IP Configuration.

- Enter:

  - IP Address: 192.168.1.2

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

For PC1:

- Click PC1.

- Open Desktop → IP Configuration.

- Enter:

  - IP Address: 192.168.2.2

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.2.1

5. Configure Router R1

Click R1, open the CLI, and enter:

enable

configure terminal

hostname R1

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 10.0.0.1 255.255.255.252

encapsulation hdlc

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

write memory

If R1 is the DCE side

Check the interface using:

show controllers serial 0/0/0

If it is the DCE side, configure a clock rate:

configure terminal

interface serial 0/0/0

clock rate 64000

end

The clock rate is configured on the DCE end of the serial connection only.

6. Configure Router R2

Click R2, open the CLI, and enter:

enable

configure terminal

hostname R2

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 10.0.0.2 255.255.255.252

encapsulation hdlc

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end

write memory

Note: Both routers must use compatible serial encapsulation settings. Cisco HDLC is typically the default encapsulation on Cisco serial interfaces, but explicitly configuring encapsulation hdlc makes the practical clear.

7. Verify HDLC Configuration

Step 1: Check the interface status

On R1 and R2, run:

show ip interface brief

Expected status:

Interface              IP-Address      Status    Protocol

Serial0/0/0             10.0.0.1        up        up

The serial interface should show up/up on both routers.

Step 2: Verify encapsulation

On R1, execute:

show interfaces serial 0/0/0

Look for output similar to:

Serial0/0/0 is up, line protocol is up

Encapsulation HDLC

This confirms that HDLC encapsulation is configured.

Step 3: Test router-to-router connectivity

From R1:

ping 10.0.0.2

Successful responses confirm connectivity between the routers.

Step 4: Test end-to-end connectivity

On PC0, open Desktop → Command Prompt and run:

ping 192.168.2.2

If the configuration is correct, PC0 should be able to communicate with PC1 through both routers.

The first ping may time out while ARP entries are being established. If that happens, run the ping again.

8. Troubleshooting

| Problem | Possible solution |
| --- | --- |
| Serial interface is down | Check the cable and execute no shutdown. |
| Interface remains down/down | Check the physical connection, module installation, and interface status. |
| Interface is up/down | Check encapsulation compatibility and DCE clock configuration. |
| Router-to-router ping fails | Verify serial IP addresses and subnet masks. |
| Router ping works, but PC-to-PC ping fails | Verify static routes, PC IP addresses, and default gateways. |
| encapsulation hdlc is rejected | Check whether the selected interface and router model support serial HDLC configuration. |

9. Result

HDLC encapsulation was configured on the serial interfaces of two routers in Cisco Packet Tracer. The configuration was verified using the show interfaces serial 0/0/0 command, and connectivity was tested using ICMP ping.

Viva Questions

- What is HDLC?

- At which OSI layer does HDLC operate?

- What is the purpose of the encapsulation hdlc command?

- What is the difference between HDLC and PPP?

- Why is a clock rate required on the DCE side?

- Which command verifies the encapsulation configured on a serial interface?

- Why are static routes required in this topology?