3 Router OSPF Demo

Now all three LANs are working:

PC1 → R1 gateway

PC2 → R2 gateway

PC3 → R3 gateway

Now we can finally test the actual OSPF operation.

Step 1 — Check R1's OSPF neighbor

On R1:

show ip ospf neighbor

We expect R2:

2.2.2.2    FULL/...

Step 2 — Check R2

On R2:

show ip ospf neighbor

We expect two neighbors:

1.1.1.1    FULL/...

3.3.3.3    FULL/...

Step 3 — Check R3

On R3:

show ip ospf neighbor

We expect:

2.2.2.2    FULL/...

If the neighbors are FULL

Then check R1's routing table:

show ip route

You should see OSPF routes similar to:

O    192.168.20.0/24

O    192.168.30.0/24

And on R3:

show ip route

you should see:

O    192.168.10.0/24

O    192.168.20.0/24

Finally — the big test

From PC1:

ping 192.168.30.10

This tests:

PC1

↓

R1

↓

R2

↓

R3

↓

PC3

If that works, you've successfully built a 3-router OSPF network and demonstrated dynamic routing end-to-end.

Router R1,R2,R3 is 2911

Start with R1 → show ip ospf neighbor

Router>show ip interface brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0 unassigned YES manual up up

GigabitEthernet0/1 192.168.10.1 YES manual up up

GigabitEthernet0/2 unassigned YES unset administratively down down

Vlan1 unassigned YES unset administratively down down

Router>

Router 2

Router>show ip interface brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0 unassigned YES manual up up

GigabitEthernet0/1 unassigned YES manual up up

GigabitEthernet0/2 192.168.20.1 YES manual up up

Vlan1 unassigned YES unset administratively down down

Router 3

Router>show ip interface brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0 unassigned YES manual up up

GigabitEthernet0/1 192.168.30.1 YES manual up up

GigabitEthernet0/2 unassigned YES unset administratively down down

Vlan1 unassigned YES unset administratively down down

PC1

IP 192.168.10.10

Subnet mask 255.255.255.0

Gateway 192.168.10.1

PC2

IP 192.168.20.10

Subnet mask 255.255.255.0

Gateway 192.168.20.1

PC3

IP 192.168.30.10

Subnet mask 255.255.255.0

Gateway 192.168.30.1

Router R1

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface gigabitEthernet 0/0

Router(config-if)#ip address 10.0.12.1 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface gigabitEthernet 0/1

Router(config-if)#ip address 192.168.10.1 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router ospf 1

Router(config-router)#router-id 1.1.1.1

Router(config-router)#network 10.0.12.0 0.0.0.3 area 0

Router(config-router)#network 192.168.10.0 0.0.0.255 area 0

Router(config-router)#exit

Router(config)#end

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#show ip interface brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0 10.0.12.1 YES manual up up

GigabitEthernet0/1 192.168.10.1 YES manual up up

GigabitEthernet0/2 unassigned YES unset administratively down down

Vlan1 unassigned YES unset administratively down down

Router 2

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

network 192.168.20.0 0.0.0.255 area 0

exit

IP Address: 192.168.30.10

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.30.1

So your three LANs become:

| Device | LAN IP |
| --- | --- |
| R1 G0/1 | 192.168.10.1/24 |
| PC1 | 192.168.10.10/24 |
| R2 G0/2 | 192.168.20.1/24 |
| PC2 | 192.168.20.10/24 |
| R3 G0/1 | 192.168.30.1/24 |
| PC3 | 192.168.30.10/24 |

2. Now we need to connect R1, R2 and R3

For a straightforward 3-router OSPF practical,

PC1

|

|

R1

|

| 10.0.12.0/30

|

R2

|

| 10.0.23.0/30

|

R3

|

|

PC3

PC2 remains connected to R2.

So the complete topology is:

PC1                 PC2                 PC3

|                   |                   |

|                   |                   |

R1 ---------------- R2 ---------------- R3

10.0.12.0/30       10.0.23.0/30

Interface addressing

We'll use:

R1

G0/0 = 10.0.12.1

G0/1 = 192.168.10.1

R2

G0/0 = 10.0.12.2

G0/1 = 10.0.23.1

G0/2 = 192.168.20.1

R3

G0/0 = 10.0.23.2

G0/1 = 192.168.30.1

This gives us:

R1 G0/0 10.0.12.1

|

| 10.0.12.0/30

|

R2 G0/0 10.0.12.2

R2 G0/1 10.0.23.1

|

| 10.0.23.0/30

|

R3 G0/0 10.0.23.2

3. Configure R1

Go to R1:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 1.1.1.1

network 10.0.12.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

exit

Then:

end

show ip interface brief

You should see approximately:

GigabitEthernet0/0   10.0.12.1       up   up

GigabitEthernet0/1   192.168.10.1    up   up

4. Configure R2

On R2:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

network 192.168.20.0 0.0.0.255 area 0

exit

Then:

end

show ip interface brief

You should get:

G0/0    10.0.12.2       up up

G0/1    10.0.23.1      up up

G0/2    192.168.20.1   up up

5. Configure R3

On R3:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.30.1 255.255.255.0

no shutdown

exit

router ospf 1

router-id 3.3.3.3

network 10.0.23.0 0.0.0.3 area 0

network 192.168.30.0 0.0.0.255 area 0

exit

Then:

end

show ip interface brief

6. Now check OSPF neighbors

This is the important part.

On R1:

show ip ospf neighbor

You should eventually see something similar to:

Neighbor ID     Pri   State           Dead Time   Address       Interface

2.2.2.2           1   FULL/DR         00:00:xx    10.0.12.2     GigabitEthernet0/0

On R2:

show ip ospf neighbor

R2 should see two neighbors:

1.1.1.1

3.3.3.3

On R3:

show ip ospf neighbor

R3 should see:

2.2.2.2

That's exactly what we expect from the chain topology:

OSPF

R1 <----------> R2 <----------> R3

|               |               |

LAN 10           LAN 20          LAN 30

7. Then test routing

Once the neighbors are FULL, check:

R1

show ip route

R1 should learn:

O    192.168.20.0/24

O    192.168.30.0/24

R2

show ip route

R2 should learn:

O    192.168.10.0/24

O    192.168.30.0/24

R3

show ip route

R3 should learn:

O    192.168.10.0/24

O    192.168.20.0/24

Then the final demonstration is:

From PC1:

ping 192.168.30.10

You should get replies:

Reply from 192.168.30.10

And you can demonstrate the complete path with:

tracert 192.168.30.10

One important point: don't start troubleshooting show ip ospf neighbor yet. First configure the physical router-to-router links and their IP addresses. Your current G0/0, G0/1, etc. output tells us the LAN interfaces are up, but it doesn't yet establish the OSPF adjacency links.

Debugging switch connections

Your PC1 configuration is correct:

IPv4 Address:    192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

So the problem is not PC1's IP configuration.

Since:

PC1 → ping 192.168.10.1 → 100% loss

and R1 also cannot ping 192.168.10.10, the problem is specifically somewhere here:

PC1 ─── Switch0 ─── R1 G0/1

Let's check the switch

Click Switch0 → CLI and run:

enable

show interfaces status

Then run:

show mac address-table

Most important

Also run on R1:

show arp

So we need three outputs:

R1

show arp

Switch0

show interfaces status

show mac address-table

Switch#show mac address-table

Mac Address Table

-------------------------------------------

Vlan    Mac Address       Type        Ports

----    -----------       --------    -----

1    0003.e4ae.e902    DYNAMIC     Gig0/1

this output gives us the clue we needed.

The switch has learned only one MAC address:

Vlan    Mac Address       Type       Ports

1       0003.e4ae.e902   DYNAMIC    Gig0/1

There is no MAC address currently learned on Fa0/1.

Given your show interfaces status:

Fa0/1   connected

Gi0/1   connected

we need to determine which physical port is connected to PC1 and which is connected to R1.

Run this on Switch0

show cdp neighbors

This should tell us which port connects to R1.

Also run:

show mac address-table dynamic

Then, on R1, run:

show arp

One important observation

Your MAC table showing:

0003.e4ae.e902    Gi0/1

is actually encouraging. It means something on Gi0/1 is actively sending Ethernet frames.

We now need to establish whether:

PC1 ── Gi0/1

R1  ── Fa0/1

or the reverse.

Please send me these two outputs:

Switch0:

show cdp neighbors

R1:

show arp

Router>

Router con0 is now available

Press RETURN to get started.

Router>sh ip int brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0 10.0.12.1 YES manual up up

GigabitEthernet0/1 192.168.10.1 YES manual up up

GigabitEthernet0/2 unassigned YES unset administratively down down

Vlan1 unassigned YES unset administratively down down

Router>ping 192.168.10.10

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.10.10, timeout is 2 seconds:

.....

Success rate is 0 percent (0/5)

Router>

Router con0 is now available

Press RETURN to get started.

Router>show arp

Protocol Address Age (min) Hardware Addr Type Interface

Internet 10.0.12.1 - 0003.E4AE.E901 ARPA GigabitEthernet0/0

Internet 10.0.12.2 137 0090.21BE.2601 ARPA GigabitEthernet0/0

Internet 192.168.10.1 - 0003.E4AE.E902 ARPA GigabitEthernet0/1

Router>show ip interface gigabitEthernet 0/1

GigabitEthernet0/1 is up, line protocol is up (connected)

Internet address is 192.168.10.1/30

Broadcast address is 255.255.255.255

Address determined by setup command

MTU is 1500 bytes

Helper address is not set

Directed broadcast forwarding is disabled

Outgoing access list is not set

Inbound access list is not set

Proxy ARP is enabled

Security level is default

Split horizon is enabled

ICMP redirects are always sent

ICMP unreachables are always sent

ICMP mask replies are never sent

IP fast switching is disabled

IP fast switching on the same interface is disabled

IP Flow switching is disabled

IP Fast switching turbo vector

IP multicast fast switching is disabled

IP multicast distributed fast switching is disabled

Router Discovery is disabled

IP output packet accounting is disabled

IP access violation accounting is disabled

TCP/IP header compression is disabled

RTP/IP header compression is disabled

Probe proxy name replies are disabled

Policy routing is disabled

Network address translation is disabled

BGP Policy Mapping is disabled

Input features: MCI Check

WCCP Redirect outbound is disabled

WCCP Redirect inbound is disabled

WCCP Redirect exclude is disabled

Router#show ip interface gigabitEthernet 0/1

GigabitEthernet0/1 is up, line protocol is up (connected)

Internet address is 192.168.10.1/30

Broadcast address is 255.255.255.255

Address determined by setup command

MTU is 1500 bytes

Helper address is not set

Directed broadcast forwarding is disabled

Outgoing access list is not set

Inbound access list is not set

Proxy ARP is enabled

Security level is default

Split horizon is enabled

ICMP redirects are always sent

ICMP unreachables are always sent

ICMP mask replies are never sent

IP fast switching is disabled

IP fast switching on the same interface is disabled

IP Flow switching is disabled

IP Fast switching turbo vector

IP multicast fast switching is disabled

IP multicast distributed fast switching is disabled

Router Discovery is disabled

IP output packet accounting is disabled

IP access violation accounting is disabled

TCP/IP header compression is disabled

RTP/IP header compression is disabled

Probe proxy name replies are disabled

Policy routing is disabled

Network address translation is disabled

BGP Policy Mapping is disabled

Input features: MCI Check

WCCP Redirect outbound is disabled

WCCP Redirect inbound is disabled

WCCP Redirect exclude is disabled

For R1 G0/1

On R1:

enable

configure terminal

interface gigabitEthernet 0/1

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

end

Then verify:

show ip interface brief

And:

show ip interface gigabitEthernet 0/1

You should now see:

Internet address is 192.168.10.1/24

not /30.

Then test PC1

Go back to PC1:

C:\>ping 192.168.10.1

You should now get:

Reply from 192.168.10.1

The first ping in Packet Tracer can occasionally time out while ARP resolves, so if necessary run it again.

Then from R1:

R1# ping 192.168.10.10

That should also work.

Important: check the other LAN interfaces too

This is something we need to be careful about because your original output said /24, but R1's actual configuration is /30.

We should verify R2 G0/2 and R3 G0/1 before proceeding.

Run:

R2

show ip interface gigabitEthernet 0/2

We want:

Internet address is 192.168.20.1/24

R3

show ip interface gigabitEthernet 0/1

We want:

Internet address is 192.168.30.1/24

Your router-to-router links, on the other hand, should remain /30:

R1-R2: 10.0.12.0/30

R2-R3: 10.0.23.0/30

So the addressing should ultimately be:

/30                  /30

R1 ---------------- R2 ---------------- R3

|                  |                  |

| /24              | /24              | /24

|                  |                  |

PC1                PC2                PC3

with:

PC1  192.168.10.10/24 → 192.168.10.1

PC2  192.168.20.10/24 → 192.168.20.1

PC3  192.168.30.10/24 → 192.168.30.1

Fix R1 G0/1 to /24 first and test ping 192.168.10.1 from PC1. This should resolve the first-hop problem.