2-router OSPF practical 9.

OSPF Practical — 2 Routers

1. Topology

Create this in Packet Tracer:

PC1 ─── Switch ─── R1 ═════════ R2 ─── Switch ─── PC2

OSPF

You can actually omit the switches and connect the PCs directly to the routers if your PT-8200 topology permits it. For a standard demonstration, use switches.

Addressing

| Device | Interface | IP Address | Network |
| --- | --- | --- | --- |
| PC1 | NIC | 192.168.1.10 | 192.168.1.0/24 |
| R1 | G0/0/0 | 192.168.1.1 | 192.168.1.0/24 |
| R1 | G0/0/1 | 10.0.0.1 | 10.0.0.0/30 |
| R2 | G0/0/0 | 10.0.0.2 | 10.0.0.0/30 |
| R2 | G0/0/1 | 192.168.2.1 | 192.168.2.0/24 |
| PC2 | NIC | 192.168.2.10 | 192.168.2.0/24 |

The important concept is that the two LANs are different networks.

192.168.1.0/24             10.0.0.0/30             192.168.2.0/24

PC1 ─── R1 ───────────────────────── R2 ─── PC2

.1        .1       .2          .1

2. Configure PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address:       192.168.1.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.1.1

3. Configure PC2

Enter:

IP Address:       192.168.2.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.2.1

4. Configure R1

Click R1 → CLI.

Enter:

enable

configure terminal

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

end

Check:

show ip interface brief

You want:

GigabitEthernet0/0/0    192.168.1.1    up    up

GigabitEthernet0/0/1    10.0.0.1       up    up

5. Configure R2

On R2:

enable

configure terminal

interface GigabitEthernet0/0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

end

Check:

show ip interface brief

You should get:

GigabitEthernet0/0/0    10.0.0.2       up    up

GigabitEthernet0/0/1    192.168.2.1    up    up

6. Test the physical/IP connectivity

Before configuring OSPF, test the directly connected link.

From R1:

ping 10.0.0.2

It should work.

From R2:

ping 10.0.0.1

It should work.

Now try from PC1:

ping 192.168.2.10

It will not work yet.

That's expected.

Why?

R1 doesn't yet know how to reach:

192.168.2.0/24

And R2 doesn't know how to reach:

192.168.1.0/24

This is where OSPF comes in.

7. Configure OSPF on R1

On R1:

enable

configure terminal

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

end

That's it.

8. Configure OSPF on R2

On R2:

enable

configure terminal

router ospf 1

router-id 2.2.2.2

network 10.0.0.0 0.0.0.3 area 0

network 192.168.2.0 0.0.0.255 area 0

end

Wait a few seconds.

You may see a message similar to:

%OSPF-5-ADJCHG: Process 1, Nbr 1.1.1.1 on GigabitEthernet0/0/0 from LOADING to FULL

FULL is what you want.

9. Check the OSPF neighbor

On R1:

show ip ospf neighbor

You should see something similar to:

Neighbor ID     State

2.2.2.2         FULL

On R2:

show ip ospf neighbor

You should see:

Neighbor ID     State

1.1.1.1         FULL

This is an important observation for your practical record:

R1 and R2 have successfully established an OSPF neighbor adjacency.

10. Check the routing table

On R1:

show ip route

You should find:

O    192.168.2.0/24

The O means:

This route was learned through OSPF.

On R2:

show ip route

You should find:

O    192.168.1.0/24

This is the key result of the practical.

11. Now perform the final test

Go to PC1 → Desktop → Command Prompt.

Run:

ping 192.168.2.10

You should now receive:

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

Your OSPF configuration is working.

The packet is travelling:

PC1

│

│ 192.168.1.10

↓

R1

│

│ 10.0.0.1 → 10.0.0.2

↓

R2

│

│ 192.168.2.1

↓

PC2

12. Demonstrate the route

From PC1:

tracert 192.168.2.10

You should see the traffic going through R1 and R2.

13. Commands to write in their practical record

Configuration

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.3 area 0

and on R2:

router ospf 1

router-id 2.2.2.2

network 10.0.0.0 0.0.0.3 area 0

network 192.168.2.0 0.0.0.255 area 0

Verification

show ip interface brief

show ip ospf neighbor

show ip route

show ip route ospf

show ip protocols

Connectivity

ping 10.0.0.2

ping 192.168.2.10

tracert 192.168.2.10

network 192.168.1.0 0.0.0.255 area 0

means:

"Enable OSPF on the interface whose IP belongs to the 192.168.1.0/24 network and advertise that network in Area 0."

And:

network 10.0.0.0 0.0.0.3 area 0

means:

"Enable OSPF on the R1–R2 link and advertise that network in Area 0."

The complete practical in one picture

10.0.0.0/30

.1                 .2

┌───┐              ┌───┐

PC1────│ R1│══════════════│ R2│────PC2

└───┘     OSPF     └───┘

.1                 .1

│                  │

192.168.1.0/24    192.168.2.0/24

│                  │

192.168.1.10      192.168.2.10

Goal: PC1 → ping 192.168.2.10 → successful.

OSPF is working correctly.

The most important line is:

O    192.168.2.0/24 [110/2] via 10.0.0.2, GigabitEthernet0/0/0

Let's understand it

O

= Route learned through OSPF.

192.168.2.0/24

= The LAN behind R2.

[110/2]

- 110 = OSPF administrative distance

- 2 = OSPF metric/cost

via 10.0.0.2

= R1 reaches that network through R2.

So R1 has automatically learned:

"To reach 192.168.2.0/24, send the packet to 10.0.0.2."

That's the core purpose of this practical.

1. Check R2's routing table

You should also run on R2:

show ip route

You should see something like:

O    192.168.1.0/24 [110/2] via 10.0.0.1

If you see that, both routers have learned each other's LAN.

2. Check PC1 configuration

On PC1:

Desktop → IP Configuration

Make sure:

IP Address:       192.168.1.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.1.1

3. Check PC2

IP Address:       192.168.2.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.2.1

4. Final test — PC1 → PC2

On PC1:

Desktop → Command Prompt

Run:

ping 192.168.2.10

You should get:

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

If the first ping fails

Don't worry if you initially get something like:

Request timed out.

Reply from 192.168.2.10

Reply from 192.168.2.10

The first packet can be lost while ARP resolution takes place. Run the ping again.

1. Interface configuration

R1 → 10.0.0.1

R2 → 10.0.0.2

2. Basic connectivity

R1#ping 10.0.0.2

!!!!!

3. OSPF configuration

router ospf 1

router-id ...

network ... area 0

4. OSPF neighbor formation

FULL/DR

5. Dynamic route learning

O 192.168.2.0/24

6. End-to-end connectivity

PC1 → R1 → R2 → PC2

Run:

show ip route ospf

On R1, it should give you the cleanest possible evidence that OSPF has learned the remote network:

O    192.168.2.0/24 [110/2] via 10.0.0.2

You can take a screenshot of:

- show ip ospf neighbor

- show ip route ospf

- Successful ping 192.168.2.10